Six Stage Pipelined

Design Implementation

Design Report

Divycharaneeswar R P             Thoutam Shiva Teja

170260018         		       170260035

Instruction Fetch Stage

- PC register: Stores the PC value of the current instruction.

- Instruction memory : Instruction Memory from which the instruction is obtained.

- ALU2	 : Used to update the PC value(PC+1 / PC+Imm)

- Branch Signal

  - 01 if BEQ , we get to know know in Execution stage 
	Temp_EX -- Immediate value to be added

  - 10 if JAL , we designed such that Imm is available from RR stage 
	Temp_RR -- Immediate value to be added

  - 00  else

- Enable read =1 always

- PC_enable = 1 if no stalling in IF stage

- PC_MUX  --Decides the PC value when the destination register is R7( Considered that on writing in R7 register also changes the PC )

  - 10 if ADD/ADI/NDU  and If the destination register is R7

  - 01 if LW/LM and If the destination register is R7

  - 11 if LHI/JLR and If the destination register is R7

  - 00 else

- PCplus1_IF is connected with input of PC

- imem_IF is connected with output of the instruction memory

- IF_ID = 1 if no stalling in IF stage

-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Instruction Decoder

In ID stage, the instruction is passed to the decoder from the  IF_ID register.Two pipeline registers ID_RR1 and ID_RR2 are used ,where ID_RR1 takes care of the LM/SM instructions.

- Decoder   :

  - Takes the input as the  instruction and gives output in 45 bits which includes

  - If decoder_enable is set, then   mem_wr_en , c flag_en ,  z flag_en are set.

Inside ID_RR

- Rf_a3_mux   : It is 1 when the instruction is LM and this mux gives the address of the register to loaded.

- PE_lmsm   :

  - This block takes the address from the immediate field and  gives output as the address of the register(pe_out) to be used in

LM  / SM instruction.

  - It also updates the address from the immediate field of already used register with 0 (pe_value)and when all 8 bits are zero in the address , the pc_zero is set as 1.

- PE_mux   :

  - This chooses between the address from the immediate field or the updated address from the PE_lmsm block

Register Read

- This stage has the forwarding unit and the register file

- FU out1 goes for alu _in1

- FU out2 goes for alu_in2

- FU_temp is data in for memory

Execution

- ALU 	   : Perform ADD, NAND operation

- Comparator : Checks the equality and generates branch signal for choosing the updated PC in IF stage

- Temp EX : Has the sign extended value  or the content of the register depending on the instruction

- ALU_out_mux: decided from the 3rd bit of  controlpoint

Memory Write/Read

- MEM 	: Data memory from which the instruction read data or writes data.

- MEM  wr en block:

  - During the store instruction if the memory stage is needed to flushed then wr_en is made 0

  - In other cases the wr_en signal from the decoder is taken

- The  wr_en_update  corresponds to  the wr_en for ADC,ADZ,NDC,NDZ instructions

WriteBack

- Enable block:

  - If flush is active for WB stage then it doesn’t write enable the zero and carry flag.

  - If there is no flush then write enable signals for carry and zero flag received from decoder is taken .

- Zero flag check:

  - WB_out gets the memory output in load case and in other cases gets the alu1 output.

  - This block checks if WB_out is zero

- C_WB comes from the carry of the alu1

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Control points:

- Wr_en mux ⇒ 0

- Rf_a3_mux ⇒ 1

- Rf_a2_mux ⇒ 2

- alu_out _mux ⇒ 3

- Mem_out_mux ⇒ 4

- MEM_WB_en ⇒ 5

Pipeline Register Contents

| Pipeline Register | Components | Length(bits) |
| --- | --- | --- |
| IFID | PC | 16 |
|  | PCplus1 | 16 |
|  | instruction | 16 |
| IDRR | Alu opcode | 1 |
|  | Op type(z/c ) | 2 |
|  | main_opcode | 4 |
|  | Control points | 6 |
|  | rf_a1 | 3 |
|  | rf_a2 | 3 |
|  | rf_a3 | 3 |
|  | pe_out | 3 |
|  | sign extended values | 16 |
|  | wr_en | 1 |
|  | mem_wr_en | 1 |
|  | carry_en | 1 |
|  | zero_en | 1 |
|  | pc | 16 |
|  | pcplus1 | 16 |
| RREX | Alu opcode | 1 |
|  | Op type(z/c ) | 2 |
|  | main_opcode | 4 |
|  | Control points | 6 |
|  | Alu input 1 | 16 |
|  | Alu input 2 | 16 |
|  | Sign extended value/rf output | 16 |
|  | PC | 16 |
|  | mem_wr_en | 1 |
|  | carry_en | 1 |
|  | zero_en | 1 |
| EXMM | Op type(z/c ) | 2 |
|  | main_opcode | 4 |
|  | Control points | 6 |
|  | mem_wr_en | 1 |
|  | zero_en | 1 |
|  | Memory address | 16 |
|  | Memory data in | 16 |
|  | pc | 16 |
| MMWB | Main _opcode | 4 |
|  | carry | 1 |
|  | Control points | 8 |
|  | Zero_en | 1 |
|  | Mem output/mem address | 16 |
|  | pc | 16 |